Modification history

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.1. |

| FWPPHAR3229 | Operate single grip harvester |
| --- | --- |
| Application | This unit of competency describes the skills and knowledge required to operate a single grip harvester for log production and use optimised production processes in a commercial forest-harvesting environment. The main job functions are to fell trees, delimb, cut stems to optimum value and segregate log products, by ensuring a safe and optimal value recovery and extraction of logs. The unit applies to individuals who work as single grip harvester operators in small or larger harvesting crews. Operators generally work under broad direction to complete routine activities related to harvesting operations and take responsibility for their work. Work health and safety legislation and environment protection legislation applies to workers in the forest harvesting industry. Requirements vary between state and territory jurisdictions. Users are advised to check with the relevant regulatory authority. |
| Prerequisite Unit | Nil |
| Unit Sector | Harvesting (HAR) |

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for operation | 1.1 Confirm, identify and report health, safety, environmental and heritage risks of operation, in line with standard operating procedures and site safety management plan 1.2 Apply risk control measures and emergency procedures, in line with standard operating procedures, site operational plan, legislative requirements and industry standards, to eliminate or minimise risks 1.3 Use production plan to identify entry points to the coupe, access to tree, extraction routes, and directions to landings, to facilitate an efficient process and minimise interference with other forestry operations 1.4 Check work area to ensure there is appropriate space to safely manoeuvre both the machine and stems, stockpile processed logs and store, or dispose of, stem waste and tops according to production plan 1.5 Assess terrain characteristics to establish machine stability 1.6 Identify cutting sequence and felling directions to minimise damage to the trees and the surrounding environment in line with the production plan 1.7 Use log cutting instructions and/or production plan to determine the product types and quality parameters to be produced 1.8 Confirm production plan targets for felling and processing and assess wood flow requirements, to ensure efficient machine operations and overall productivity 1.9 Carry out pre-operational checks on machine in line with organisational procedures or manufacturer's recommendations, to ensure optimal production performance and operational effectiveness |
| 2. Operate single grip harvester | 2.1 Use industry-accepted warning signs, vehicle-to-vehicle communication systems and procedures, to maintain communication and workplace safety during operation 2.2 Assess individual trees to determine the best machine position, fall direction, hazards, placement of cuts, and grade quality 2.3 Drive and position the machine and harvester head in response to tree characteristics and terrain conditions, to ensure stability 2.4 Use appropriate methods to reset the harvesting head length counter, to prevent loss of log volume or value 2.5 Assess tree species, size and quality characteristics, in line with log cutting instructions and/or production plan requirements, and use cabin controls to clamp, cut and produce logs with optimal length, quality and diameter 2.6 Adjust clamping, cutting and processing techniques in response to unexpected characteristics, movement or condition of tree, to minimise log and log surface damage in line with the production plan requirements 2.7 Move machine over pre-arranged routes to minimise environmental impact |
| 3. Segregate and stack logs | 3.1 Visually assess logs for size, species and quality characteristics during stacking at the pre-bunching site, to ensure they are placed in the appropriate stack for transport and further processing and in line with the production plan requirements 3.2 Mark and report logs that cannot be clearly and safely assessed, in line with the production plan requirements 3.3 Position and construct log stacks, to avoid log damage provide stability, restrict height for efficient extraction and allow space for the conduct of other operations in line with worksite layout and restrict height for efficient extraction in line with the production plan requirements 3.4 Maintain stack separation and identification, to ensure products are presented for an optimum transport and logistics cycle time and are in line with industry standards and production plan requirements |
| 4. Carry out machine operator maintenance | 4.1 Park, shut-down, de-energise and isolate single grip harvester, in line with organisational procedures or manufacturer’s recommendations 4.2 Adjust feed roller pressures and delimbing knife settings to minimise roller stud and delimbing damage 4.3 Clean, check and carry out routine maintenance tasks on single grip harvester to maintain optimum performance 4.4 Identify and report machine faults, malfunctions or problems, in line with organisational procedures or manufacturer's recommendations 4.5 Clean work area and dispose of, or recycle, waste materials, in line with site environmental management plan 4.6 Record single grip harvester operations and maintenance activities |

| Foundation Skills This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria. |  |
| --- | --- |
| Skill | Description |
| Learning | - develop knowledge of log species, quality characteristics, defects and product uses |
| Reading | - interpret production plans, site safety and environmental management plans in a forest harvesting environment - interpret standard operating procedures and/or manufacturers’ instructions for the operation of forest harvesting machinery |
| Writing | - record operation, production and machine maintenance on paper based or electronic media |
| Oral Communication | - interact verbally and non-verbally using agreed systems and processes to communicate with others on site to ensure safe and effective single grip harvester operations |
| Numeracy | - interpret gauges relevant to safe operation of harvesting machinery - determine construction of stable log stack |
| Navigate the world of work | - identify and describe own skills, knowledge and experience within context of job role - understand main tasks, responsibilities and boundaries of own role - operate single grip harvester according to site standard operating procedures, relevant to own responsibilities |
| Interact with others | - use appropriate vocabulary, including technical language directly relevant to role - use modes of communication suitable to purpose to confirm and clarify understanding - communicate and report operational and safety information to on-site personnel |
| Get the work done | - identify and resolve equipment issues and faults - recognise and respond to routine problems - take responsibility for planning, sequencing and prioritising tasks and own workload for efficiency and effective outcomes |

| Unit Mapping Information |  |  |  |
| --- | --- | --- | --- |
| Code and title current version | Code and title previous version | Comments | Equivalence status |
| FWPHAR3229 Operate single grip harvester | FWPHAR3214 Operate a single grip harvester | Optimisation processes and practices included | No equivalent unit |

| Links | Companion Volume Implementation Guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingPackages.aspx/Pages/Home.aspx |
| --- | --- |

| TITLE | Assessment requirements for FWPHAR3229 Operate single grip harvester |
| --- | --- |
| Performance Evidence |  |
| An individual demonstrating competency must satisfy all the elements, performance criteria and foundation skills of this unit. There must be evidence that, on at least one occasion, the individual has: planned, prepared and safely and effectively operated a single gip harvester to cut and produce log products with optimal length, quality and diameter, in line with log cutting instructions, quality specifications or production plan checked risks associated with the operation and applied standard operating procedures followed production plan requirements, log cutting instructions or quality specifications and site safety and environmental management plan adjusted machine driving and positioning techniques in response to terrain conditions and unexpected characteristics, movement or condition of tree applied felling and processing techniques to minimise damage to the stem, logs, standing trees, machine and surrounding environment in line with the production plan requirements applied knowledge of log species, quality characteristics and defects during operation, in line with the production plan requirements placed, stacked and presented logs for efficient transportation and identification, in line with the production plan requirements communicated effectively with others in the work area shut down, de-energised and isolated machine in line with organisational procedures and operator’s manual carried out routine maintenance on single grip harvester. |  |

Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of: - typical information and requirements contained in a production plan, log cutting instructions or product quality specifications, and site safety and environmental management plan - hazardous or safe working zones in forest harvesting operations - health and operational hazards or risks related to single grip harvesting operations and factors that can change or introduce new hazards or risks, including characteristics of difficult trees and risks associated with falling them safely - industry-accepted work health and safety risk control measures, legal obligations of the current Work Health and Safety Acts or Occupational Health and Safety Acts and regulations, and requirements of industry standards and/or codes of practice for single grip harvesting operations, as outlined in organisational procedures and production plan - industry-accepted warning signs, their correct positioning, vehicle-to-vehicle communication systems and procedures to maintain communication and work safety during single grip harvesting operations - potential impacts of single grip harvester operations on onsite and offsite environmental values, including cultural heritage, flora, fauna, ground compaction, water contamination and soil erosion - industry-accepted environmental control measures, restrictions, legal obligations of the current Environment Protection Acts and regulations and requirements of industry standards and/or codes of practice for single grip harvesting operations, as outlined in organisational procedures and production plan - typical location of loading and/or landing site - tree felling patterns - extraction methods - wood flow requirements and extraction and production bottlenecks particular to a production system - tree shapes, species, quality characteristics and log defects particular to the site and production system - regional quality requirements for log products with particular end uses, markets and values - requirements and methods to achieve maximum stump height in response to unexpected characteristics or condition of the trees and ground level - maximum log damage and defect and minimum quality allowances particular to a production system - techniques particular to a production system to avoid mechanical damage to logs during felling, processing and stacking; achieve saw cuts at both ends of the product that meet quality specifications; avoid incorrect or excessive log butting; and separate and present logs for efficient transportation and identification of the product - effects of weather on ground conditions - effects of slope and terrain conditions on machine stability and operation - machine specifications, restrictions and safe operating procedures, including stability and operation in different terrain conditions, slopes and road types, chain shot rules, methods to shut down, de-energise and isolate the machine and optimal direction of travel, as outlined in organisational procedures or operator’s manual - using machine cabin controls - techniques for safe and efficient driving of single grip harvester - location of machine’s major mechanical systems and the energy they store - methods to conduct machine pre-operational checks and detect faults, as outlined in organisational procedures or operator’s manual - firefighting equipment requirements - machine maintenance and servicing requirements, as outlined in organisational procedures or operator’s manual - industry-accepted reporting requirements regarding site, incident and hazard, personal reporting, check-in, production, machine maintenance, block completion.

Assessment Conditions

Assessment of this unit of competency must take place under the following conditions: - physical conditions: skills must be demonstrated in a forest harvesting site or an environment that accurately represents workplace conditions - resources, equipment and materials: single grip harvester suitable trees to fell and process personal protective equipment required in harvesting operations vehicle-to-vehicle communication system - specifications: access to organisational policies and procedure which cover off on current Work Health and Safety Acts or Occupation Health and Safety Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations access to organisational policies and procedure which cover off on current Environment Protection Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations access to production plan, site operational plan, log cutting instructions and safety and environmental management plan access to site emergency procedure access to organisational standard operating procedures for single grip harvester or operator’s manual access to user manual for vehicle-to-vehicle communication system. Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

| Links | Companion Volume Implementation Guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingPackages.aspx/Pages/Home.aspx |
| --- | --- |